# SIH PS — Adaptive Underwater Sonar

## 1. Core Problem: Underwater Acoustic Trade-Off

Underwater acoustics has a nasty trade-off:

* **Higher frequency**

→ generally better spatial/detail resolution

→ higher propagation loss

* **Lower frequency**

→ generally better range/coverage

→ less fine detail

A 2026 side-scan sonar study explicitly describes this trade-off: its high-frequency sonar produced finer target detail but suffered stronger attenuation and reduced useful range, while lower frequency provided wider coverage with poorer visibility of small targets.

### Example Scenario

Imagine your AUV moving through:

**Clear shallow water → deeper water → sediment-rich water → noisy/reverberant region**

A waveform that worked well in the first region may not remain optimal in the next.

### Core Problem

That is the core problem.

Do **not** present adaptive sonar as:

> “The AI chooses a frequency.”

That is too shallow.

Your system should continuously answer:

> **Given the current underwater condition, what waveform gives me enough detection performance for the minimum required energy?**

---

# 2. Parameters That Need to Adapt

## 2.1 Centre Frequency / Bandwidth

This controls the **range-resolution trade-off**.

### Good Conditions

→ Higher centre frequency / wider useful bandwidth

→ More detail

### Poor Propagation Conditions

→ Lower frequency / different bandwidth

→ Sacrifice some detail for penetration/range

The SIH statement explicitly defines this as one of the parameters to adapt.

---

## 2.2 Pulse Duration

This controls how much **acoustic energy** is put into the environment.

### Longer Pulse

**Advantages:**

* More transmitted energy

* Potentially better detection/range

**Disadvantages:**

* More transmission time

* More energy consumption

* Potentially poorer range resolution unless pulse compression is used

### Why LFM Chirps Matter

This is one reason **LFM chirps** are important.

An LFM chirp spreads frequency over a longer pulse but can later be pulse-compressed during processing, allowing a system to get useful energy without simply using an extremely short, extremely high-power pulse.

Research on active sonar waveform design discusses this long-duration/wide-bandwidth pulse-compression approach.

---

## 2.3 Amplitude / Transmit Power

Do **not** blindly transmit maximum power.

### Easy Environment

Lower amplitude

→ Enough SNR

→ Save energy

### Difficult Environment

Increase amplitude

→ Recover detection margin

The SIH statement specifically asks for **amplitude/signal power adaptation**.

This is where your **energy-aware argument** becomes strong.

---

# 3. Environmental Factors

Temperature, salinity and depth affect the **speed of sound and acoustic propagation characteristics**.

A 2025 underwater-acoustic study explicitly uses:

* Temperature

* Salinity

* Depth

as environmental variables affecting underwater acoustic-channel behaviour.

### Frequency

Frequency also matters enormously because acoustic absorption increases with frequency.

The 2026 side-scan study states that propagation loss is strongly frequency-dependent and that higher-frequency signals have larger attenuation coefficients in water.

---

# 4. Real Physical Picture

The real physical picture is:

**Frequency + temperature + salinity + depth + scattering environment + reverberation + ambient noise + target + geometry**

all influence what waveform is useful.

---

# 5. Problem 1 — Existing Sonar Systems Are Often Configurable, but Not Truly Continuously Adaptive

Modern sonar can have multiple operating frequencies.

For example, commercial/field systems may provide multiple selectable frequencies.

But there is a major difference between:

### Configuration

> “The operator can select 400 kHz or 900 kHz.”

and:

### Adaptation

> “The AUV senses the environment and automatically changes the waveform while operating.”

The first is **configuration**.

The second is **adaptation**.

Recent software-defined-sonar research is explicitly moving toward the second concept.

Zhou et al. proposed a 2025 software-defined sonar architecture for unmanned underwater systems because conventional sonar installations create separate hardware, energy and interface burdens and lack the flexibility desired for multifunctional unmanned systems.

### Your Response

Build:

**Environment sensing → Decision logic → Waveform parameter update → Hardware waveform generation**

with **no operator intervention**.

---

# 6. Problem 2 — Software-Defined Sonar Still Has a Hardware Limitation

This is a very important research finding.

A 2025 paper on software-defined sonar does **not** claim software can magically make any sonar operate at any frequency.

It explicitly says the sonar frequency remains constrained by:

* Usable bandwidth

* Transducer

* Data-acquisition electronics

* Power amplifier

and these components have to match the transducer.

If you substantially change the operating frequency, the transducer and transmit/receive electronics may also need modification.

---

# 7. The Hardware Constraint

In simple language:

You cannot say:

> “Our software supports 100 kHz–500 kHz.”

and assume the hardware can physically produce equally good acoustic output across that entire range.

The chain is:

**Digital waveform**

↓

**DAC**

↓

**Amplifier**

↓

**Matching network**

↓

**Transducer**

↓

**Water**

Every stage has:

* Bandwidth limitations

* Efficiency limitations

---

# 8. Therefore: Hardware-Aware Software-Defined Sonar

Your “software-defined” system actually needs to be:

> **Hardware-aware software-defined sonar**

This is much stronger.

---

# 9. Research Gap

There is a recurring research pattern:

* Adaptive intelligence exists.

* Digital sonar hardware exists.

* Low-power embedded hardware exists.

But the challenge is:

> **Integrating these pieces into one physically demonstrated, real-time, power-conscious transmitter.**

That is the gap you should attack.

---

# 10. Proposed System Architecture

```text

UNDERWATER ENVIRONMENT

↓

┌───────────────────────────┐

│   Environmental Inputs    │

│                           │

│ T / S / Depth / Noise     │

│ / Turbidity proxy         │

└─────────────┬─────────────┘

↓

ADC / Sensor Interface

↓

┌───────────────────────────┐

│    Adaptation Controller  │

│                           │

│ Select:                   │

│ • Centre frequency       │

│ • Bandwidth              │

│ • Pulse duration         │

│ • Amplitude              │

└─────────────┬─────────────┘

↓

┌───────────────────────────┐

│     Waveform Generator    │

│                           │

│ LFM / sweep / phase       │

│ coded pulse               │

└─────────────┬─────────────┘

↓

DMA + Timer

↓

DAC

↓

Analog Filter

↓

Power Amplifier

↓

Matching Network

↓

Transducer

↓

ACOUSTIC PING

```

---

# 11. Critical Limitation: Software-Defined ≠ Infinitely Frequency-Flexible

More importantly, that paper exposes an extremely important limitation:

> **Software-defined does not mean infinitely frequency-flexible.**

The operating frequency is constrained by:

* Transducer bandwidth

* Data-conversion electronics

* Power amplifier

Large changes in operating frequency can require hardware changes.

This is a real gap.

---

# 12. What Student Projects May Get Wrong

This is where a lot of student projects will bullshit themselves.

They will demonstrate:

> “The FFT moved from 100 kHz to 500 kHz.”

That proves the **electrical waveform generator changed**.

It does **not** prove the sonar transmitter is actually transmitting those frequencies effectively underwater.

### Critical Distinction

**Electrical waveform generation ≠ effective underwater acoustic transmission**

That distinction is critical.

---

# 13. Level 3 — Hardware-Aware Real-Time Adaptation

This is where your real opportunity lies.

The strongest version of your project is:

> **The adaptation algorithm is constrained by the measured characteristics of the physical transmitter chain.**

---

# 14. Stronger Version

### Embedded Waveform Engine

*

### DMA

*

### Timer-Driven DAC

*

### Hardware Filtering

*

### Adaptive Parameter Selection

*

### Energy Model

*

### Hardware Characterization

---

# 15. Very Strong Version

> **Embedded adaptive transmitter + measured transducer/electronics envelope + acoustic/hydrophone feedback + closed-loop waveform selection + energy/performance benchmarking**

That is the level I would target for a **national-stage engineering prototype**.

---

# 16. Biggest Revelation from the Research

Your competitors do not necessarily have to beat you with a better algorithm.

They could beat you by demonstrating:

> “Our electrical waveform looks nice.”

No.

Your team should demonstrate:

> **“We know exactly what the transmitter is physically capable of, and our adaptation algorithm respects those limits.”**

That's engineering maturity.

The 2025 software-defined sonar work makes this transducer/electronics constraint explicit.

---

# 17. Commercial Systems Make Your Novelty Harder

This is the part I would put in your team's internal warning document.

There are already commercial systems with:

* Multiple frequencies

* Adaptive beamforming

* Dynamic focusing

* Adaptive waveform features

* Compact AUV integration

* Low-power operation

For example:

* Klein's MANTIS advertises adaptive beamforming and dynamic focusing.

* Another commercial side-scan system advertises adaptive waveform adjustment.

Therefore:

## Do NOT claim:

> “Existing sonar cannot adapt.”

Instead:

> **“Existing systems demonstrate pieces of adaptive and software-defined sonar, but we are targeting a compact, low-power, open embedded transmitter architecture whose adaptation, waveform synthesis and analog output are demonstrated and quantitatively characterized on accessible hardware.”**

That is defensible.